COMPUTER NETWORKS PRACTICAL FILE

Practical 15 & 16

Student Name: Archita Arora

Registration Number: RA2411003030275

Subject: Computer Networks

Software Used: Cisco Packet Tracer

# Practical 15: Configuration and Verification of RIPv2

## 1. Aim

To configure RIPv2 dynamic routing on three routers and verify communication between different LANs using a classless VLSM-based addressing scheme.

## 2. Network Topology

The network consists of three routers connected through two serial links. Each router provides connectivity to one local PC/LAN.

PC0 → R0 FastEthernet2/0

R0 Serial0/0 → R1 Serial1/0

PC2 → R1 FastEthernet2/0

R1 Serial0/0 → R2 Serial1/0

PC4 → R2 FastEthernet2/0

Note: The interface names may vary according to the router model selected in Packet Tracer. The DCE side of each serial connection should be identified before applying the clock rate.

## 3. IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Prefix | Gateway |
| --- | --- | --- | --- | --- | --- |
| R0 | FastEthernet2/0 | 192.168.10.1 | 255.255.255.224 | /27 | — |
| R0 | Serial0/0 | 192.168.10.97 | 255.255.255.252 | /30 | — |
| R1 | FastEthernet2/0 | 192.168.10.33 | 255.255.255.224 | /27 | — |
| R1 | Serial1/0 | 192.168.10.98 | 255.255.255.252 | /30 | — |
| R1 | Serial0/0 | 192.168.10.101 | 255.255.255.252 | /30 | — |
| R2 | FastEthernet2/0 | 192.168.10.65 | 255.255.255.224 | /27 | — |
| R2 | Serial1/0 | 192.168.10.102 | 255.255.255.252 | /30 | — |
| PC0 | FastEthernet0 | 192.168.10.2 | 255.255.255.224 | /27 | 192.168.10.1 |
| PC2 | FastEthernet0 | 192.168.10.34 | 255.255.255.224 | /27 | 192.168.10.33 |
| PC4 | FastEthernet0 | 192.168.10.66 | 255.255.255.224 | /27 | 192.168.10.65 |

LAN subnets used are 192.168.10.0/27, 192.168.10.32/27 and 192.168.10.64/27. The two point-to-point serial networks are 192.168.10.96/30 and 192.168.10.100/30.

## 4. RIPv2 Router Configuration

### R0

enable
configure terminal
hostname R0
interface fastEthernet2/0
 ip address 192.168.10.1 255.255.255.224
 no shutdown
exit
interface serial0/0
 ip address 192.168.10.97 255.255.255.252
 clock rate 64000
 no shutdown
exit
router rip
 version 2
 no auto-summary
 network 192.168.10.0
end
write memory

### R1

enable
configure terminal
hostname R1
interface fastEthernet2/0
 ip address 192.168.10.33 255.255.255.224
 no shutdown
exit
interface serial1/0
 ip address 192.168.10.98 255.255.255.252
 no shutdown
exit
interface serial0/0
 ip address 192.168.10.101 255.255.255.252
 clock rate 64000
 no shutdown
exit
router rip
 version 2
 no auto-summary
 network 192.168.10.0
end
write memory

### R2

enable
configure terminal
hostname R2
interface fastEthernet2/0
 ip address 192.168.10.65 255.255.255.224
 no shutdown
exit
interface serial1/0
 ip address 192.168.10.102 255.255.255.252
 no shutdown
exit
router rip
 version 2
 no auto-summary
 network 192.168.10.0
end
write memory

## 5. Verification of RIPv2

Run the following commands on each router:

show ip interface brief

show ip protocols

show ip route

Test connectivity from PC0 to the other two LANs:

ping 192.168.10.34

ping 192.168.10.66

Observation: After routing information is exchanged, remote networks should appear as RIP-learned routes (R) in the routing table, while local networks remain directly connected (C).

[Paste your Packet Tracer topology screenshot here]

[Paste your successful ping / show ip route screenshots here]

# Practical 16: Single-Area OSPFv2 Configuration

## 1. Aim

To configure OSPF version 2 in a single Area 0 network, assign unique router IDs, advertise the required interfaces using wildcard masks, and verify the resulting neighbor relationships and routes.

## 2. OSPF Network Information

| Router | Network | Wildcard Mask | Area | Router ID |
| --- | --- | --- | --- | --- |
| R0 | 192.168.10.0 | 0.0.0.31 | 0 | 10.10.10.1 |
| R0 | 192.168.10.96 | 0.0.0.3 | 0 | 10.10.10.1 |
| R1 | 192.168.10.32 | 0.0.0.31 | 0 | 20.20.20.1 |
| R1 | 192.168.10.96 | 0.0.0.3 | 0 | 20.20.20.1 |
| R1 | 192.168.10.100 | 0.0.0.3 | 0 | 20.20.20.1 |
| R2 | 192.168.10.64 | 0.0.0.31 | 0 | 30.30.30.1 |
| R2 | 192.168.10.100 | 0.0.0.3 | 0 | 30.30.30.1 |

## 3. OSPF Router Configuration

The interface addressing from Practical 15 is retained. RIPv2 is removed and OSPF process 1 is enabled in Area 0.

### R0

enable
configure terminal
no router rip
router ospf 1
 router-id 10.10.10.1
 network 192.168.10.0 0.0.0.31 area 0
 network 192.168.10.96 0.0.0.3 area 0
end
write memory

### R1

enable
configure terminal
no router rip
router ospf 1
 router-id 20.20.20.1
 network 192.168.10.32 0.0.0.31 area 0
 network 192.168.10.96 0.0.0.3 area 0
 network 192.168.10.100 0.0.0.3 area 0
end
write memory

### R2

enable
configure terminal
no router rip
router ospf 1
 router-id 30.30.30.1
 network 192.168.10.64 0.0.0.31 area 0
 network 192.168.10.100 0.0.0.3 area 0
end
write memory

If the router ID is changed after OSPF has already formed, restart the OSPF process using 'clear ip ospf process' and confirm the prompt, or restart the router so that the new router ID becomes active.

## 4. OSPF Verification

Use these commands on the routers:

show ip ospf neighbor

show ip route

show ip protocols

From PC0, check connectivity with PC2 and PC4:

ping 192.168.10.34

ping 192.168.10.66

Observation: OSPF-learned routes should be identified by O in the routing table, while directly connected networks are shown using C.

[Paste your OSPF neighbor and routing-table screenshots here]

[Paste your successful ping screenshots here]

## 5. Result / Conclusion

RIPv2 and single-area OSPFv2 were configured on the three-router topology. The addressing plan was implemented using separate LAN and point-to-point subnets. Connectivity between the remote PCs was tested using ping. The routing tables were inspected to identify dynamically learned routes. For the final submission, the observations in this section should be updated according to the actual Packet Tracer results obtained during testing.